CITY OF BELFAST HARBOR DEPARTMENT

OTHER THAN INNER HARBOR 

MOORING INSPECTION REPORT
MOORING #_________DATE______________ LOCATION COORDINATES  N______ ______ ______

                                                


   


        W______ ______ ______

OWNER OF MOORING_______________________________

SET/INSPECTED BY_________________________________SIGNATURE______________________




INSPECTION:

PASSED__________    FAILED__________


Sec. 30-168.  Inspections


Every two calendar years a mooring shall either be lifted or inspected



by a diver at the expense of the owner and at the direction of the 

harbormaster 


Sec. 30-167 Inner Harbor Design Standards


1) All moorings shall consist of a granite block or other anchor of 

    sufficient weight to hold the vessel for which they are to be used. 


2) Hand-mixed concrete blocks, old engine blocks and other 

    miscellaneous weighted objects are not acceptable as mooring 

    anchors in Belfast Inner Harbor.  


3) The moorings shall have heavy steel bottom chain of a minimum 

    as stated in standards.


4) The minimum mooring scope shall be approximately two times 

    the water depth at mean high water. Total scope shall include

    bottom chain and lighter gauge top chain with a minimum 

    diameter, as stated in standards, each of which shall consist of 

    approximately one-half of the total length.


5) Each mooring must have at least one swivel at the connection either 

    between the top chain and mooring buoy or between the heavy 

    bottom chain and the top chain. All swivels and shackles must be 

    appropriate size for the chain size.


6) Pennants connecting the mooring buoy to the moored boat shall be 

    fastened to the top chain or top swivel and shall consist of nylon, or 

    other appropriate pendant line equal in length up to one-half the length 

    of the vessel. Refer to minimum standards.
7) Despite the dimension standards established in this section, any part of

    a mooring showing excessive or obvious wear or any mooring which 

    does not meet the harbormaster's approval shall not be permitted.

Minimum (fair weather) standards for areas other than Inner Harbor

  Length of          Granite or 
  Mushroom              Top Chain    
    Bottom Chain         
    Pendant

Vessel (ft.)        Equiv. (lbs.)              (lbs.)                     (inches)                         (inches)                     (inches)

0--9
           
      500 
       150 
 
 3/8
      under 20 feet w/approval             1/2

    10--19         
    1000

       200
 
 3/8
 
  
7/16
                    5/8

    20--30         
    1500

       300
 
 1/2
 
   
5/8
                    3/4

    31--35         
    2000

       350
 
 1/2
               
5/8
                    3/4

    36--40         
    2500

       450
              1/2
 
   
3/4
                     1

    41--50         
    3000
                    500
 
 1/2
               
7/8    
        
        1

    51 and over w/approval

PENDANT





  Length______________


 


  Size _______________





TOP CHAIN





  Length______________


 


  Size________________





SWIVEL


  


  Top Size_____________





  Middle Size___________





BOTTOM CHAIN


  


  Length______________


 


  Size________________





MOORING ANCHOR





  Granite___





  Cement___





WEIGHT______________








